Exceptional Covered Spaces™

PR-20-65-78-58

A&S Landscape

Motiva Entro™ Canopy

Specification

aandslandscape.co.uk

# Document Control Sheet

| Issued by: | A&S Landscape Battlefield Enterprise Park Shrewsbury SY1 3JE | T: 01743 444100 E: sales@aandslandscape.com |
| --- | --- | --- |
| Author: | Miss Lisa Walker Head of Architectural Projects | T: 01743 444100 E: lisa@aandslandscape.com |
| Issue date: | Tuesday, 07 May 2024 |  |
| Project client: |  |  |
| Project title: |  |  |
| Publication status: | Outline |  |
| Publication revision: | 0 |  |
| Project description: | External canopy |  |

Contents

Document Control Sheet	0

B91 Buildings in the landscape	1

B91
Buildings in the landscape

Clauses

2		To be read with preliminaries/ general conditions.

System outline

105		Proprietary small buildings/ structures

- Description: Generally

- Building type: Canopy

- Services requirements: Rainwater drainage systems - Gutters and downpipes

System performance

220	Durability

- Description: Of canopy - framework and finishes

- Minimum life span

  - Fabric

    - Element: Framework

    - Duration: 25 years

  - Finishes: Roof covering

    - Item: Polycarbonate sheeting

    - Duration: 15 years

220A Durability

- Description: Of canopy - rainwater disposal

- Minimum life span

  - Fabric

    - Element: Rainwater Goods - Generally

    - Duration: 25 years

  - Finishes: Rainwater Goods

    - Item: Rainwater pipes, hoppers and guttering

    - Duration: 25 years

240	Water penetration

- Requirement: Under site exposure conditions, moisture must not penetrate onto internal surfaces or into cavities not designed to be wetted.

Products

330	Canopy

- Description: Generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T: 01743 444100, E: sales@aandslandscape.com

  - Product reference: Motiva Entro™ Canopy

- Size: Refer to Project specific drawings

- Frame: Carbon steel - With either galvanized or Dura-coat™ colour finish

- Panels: Various infill options - Refer to project specific drawings and details

- Roof covering: Polycarbonate - Clear, Opal or Coloured

- Accessibility: No requirement

- Accessories/ Special requirements: Wall brackets, Post Pads - Colours - varies

- Method of fixing to ground, base or walls: As project specific drawings

335	Covered walkway

- Description: Generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T:01743 444100,E: sales@aandslandscape.com

  - Product reference: Motiva Entro™ Canopy

- Size: Refer to Project specific drawings

- Frame: Carbon steel - With either galvanized or Dura-coat™ colour finish

- Panels: Various infill options - Refer to project specific drawings and details

- Roof covering: Polycarbonate - Clear, Opal or Coloured

- Accessibility: No requirement

- Accessories/ Special requirements: Wall brackets, Post Pads - Colours - varies

- Method of fixing to ground or base: As project specific drawings

385	Shelter

- Description: Generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T:01743 444100, E:sales@aandslandscape.com

- Product reference:  Motiva Entro™ Canopy

- Size: Refer to project specific drawings

- Frame: Carbon steel - With either galvanized or Dura-coat™ colour finish

- Panels: Various infill options - Refer to project specific drawings and details

- Roof covering: Polycarbonate - Clear, Opal or Coloured

- Accessibility: No requirement

- Accessories/ Special requirements: Wall brackets, Post Pads - Colours - varies

- Method of fixing to ground or base: As project specific drawings

420	Waiting shelter

- Description: Generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T: 01743 444100, E: sales@aandslandscape.com

  - Product reference: Motiva Entro™ Canopy

- Size: Refer to project specific drawings

- Frame: Carbon steel - With either galvanized or Dura-coat™ colour finish

- Panels: Various infill options - Refer to project specific drawings and details

- Roof covering: Polycarbonate - Clear, Opal or Coloured

- Accessibility: No requirement

- Accessories/ Special requirements: Wall brackets, Post Pads - Colours - varies

- Method of fixing to ground or base: As project specific drawings

Materials

430	Aluminium

- Description: Gutters and rainwater pipework - generally

- Standards: To relevant parts of BS EN 515 and BS EN 12020.

  - Mechanical properties of extruded rod/ bar, tube and profiles: To BS EN 755-2.

  - Tolerances on dimensions and form of seamless tubes: To BS EN 755-7.

  - Profiles, tolerances on dimensions and form: To BS EN 755-9.

  - Structural members: In accordance with BS EN 1999-1.1.

  - Aluminium sheet: To BS EN 573-3.

- Alloy, temper and thickness: Suitable for the application and specified finish.

- Finish: Dura-coat™ colour finish

  - Colour: Varies - Refer to project specific drawings

- Method of fixing: As manufacturer's recommendations

- Additional requirements: None

445	Metal profile

- Description: Canopy framework - generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T: 01743 444100, E:sales@aandslandscape.com

  - Product reference: Motiva Entro™ Canopy

- Material: Steel to BS 3083 and BS EN 508-1

  - Thickness (nominal): As project specific drawings

  - Profile: As project specific drawings

  - Finish: Galvanized or Dura-coat™ colour finish

    - Colour: To BS 4800 - Colour varies

- Method of fixing: As manufacturer's recommendations

- Additional requirements: Alu-tuff™ Aluminium guttering and downpipes

480	Glazing plastics

- Description: To canopies - generally

- Manufacturer: A&S Landscape, March Way, Battlefield Enterprise Park, Shrewsbury, Shropshire, SY1 3JE, United Kingdom, T: 01743 444100, E: sales@aandslandscape.com

  - Product reference: Motiva Entro™ Canopy

- Standards: To relevant parts of BS 6262-4.

  - Impact strength: To BS 5544, BS 6206, BS EN 356 and BS EN 12600.

- Material: Polycarbonate

- Thickness: Refer to project specific drawings

- Finish/ Colour: Clear, Opal or project specific standard colour - TBC

- Perimeters: Manufacturer's standard

- Method of fixing: As manufacturer's recommendations

- Additional requirements: Roof panels: System 60 structured roofing panel in giving thermal transmittance U-Value: 2.2 W/m, Noise reduction: 20dB (DIN 52210-75 Rw).Nonfragility rating: Class ‘B’ to ACR (M) 001:2005 Fire classification: Euroclass - EN13501-1 2006 B-s1, d0 (as UK Class 0)

Execution/ erection / installation

600	Erection / installation generally

- Frameworks: Assemble and brace, including temporary members required for installation.

- Contact between dissimilar metals: Avoid.

- Fixings: Fully bolt together. Tighten bolts.

- Temporary support: Do not subject members to non-design loadings.

610	 Concrete foundations generally

- Concrete: To BS 8500-2.

- Mix: Designated concrete not less than GEN 1 or standard prescribed concrete not less than ST2.

- Admixtures: Do not use.

- Depth of foundations, bedding, haunching: Appropriate to provide adequate support and to receive overlying soft landscape or paving finishes.

- Foundation holes: Neat vertical sides.

- Components: Accurately positioned and securely supported.

- Concrete fill: Compact as filling proceeds.

- Temporary component support: Maintain undisturbed for minimum 48 hours.

615	 Setting components in earth

- Holes: Excavated by appropriate means to size on site specific drawings.

- Components: Position accurately and support securely.

- Buried depth (minimum): Refer to project specific drawings

620	Building in to masonry walls

- Components being built in: Position accurately and support securely. Set in mortar and pointed neatly to match adjacent walling.

- Temporary support: Maintain for 48 hours (minimum) and prevent disturbance.

625	Electrical and data services

- Services connection required: Power for lighting and heating.

- Standard: To BS 7671.

- Coordinate with services trades.

630	Erection of prefabricated buildings/ structures

- Checking: Five days (minimum) before proposed erection date, check foundations, holding down bolts, etc.

- Inaccuracies or defects in prepared bases or supplied buildings/ structures: Report immediately. Obtain instructions before proceeding.

Completion - Not Used

Ω End of Section

A&S Landscape

T : 01743 444100

E : sales@aandslandscape.com

Head Office

A&S Landscape

Battlefield Enterprise Park

Shrewsbury

SY1 3JE

aandslandscape.co.uk

Specification created using NBS Chorus

aandslandscape.co.uk